# Recording Main Incoming Message on an Aspire  NVM-2e

## From any System Telephone

- Press the CALL1 button.

- Lift Handset.

- Dial 700 & wait for Voice Mail to answer.

- Enter mailbox 301 upon request from Voice Mail.

- Enter security code if requested.

- Press S A to enter System Administrator program.

- Press I for Instruction message mailbox.

- Enter Mailbox number 802 for first recording.

  - There may be more than one recording. The typical application would be 802 for Daytime message, 803 for Nighttime and possibly 804 as a directory listing of personnel.

- Press L to listen to existing recording.

- Press # to exit listen mode.

- Press R to record new message.

- Press # when finished.

  - Helpful hint. If you mess up while recording press # R to redo recording. There is no need to hang up and go all the way back to the beginning.

13. Hang up.